產 品 規 格 承 認 書

SPECIFICATION FOR APPROVAL

| 客戶名稱 CUSTOMER |  |
| --- | --- |
| 產品名稱 PRODUCT NAME | 玻璃慢断保险丝 |
| 型號規格 MODEL DETAIL | 3GTU |
| 承认日期 APPROVAL DATE |  |

| 編制 | 經理 | 業務 |
| --- | --- | --- |
| PREPARED | MANAGER | OPERATION |
| 陈兰欣 | 陈奕霖 |  |

| 承認 | 審核 | 批准 |
| --- | --- | --- |
| SIGNATURE | CHECKED BY | APPROVAL BY |

1.規格型號和安規認證

MODEL NUMBER AND SAFETY CERTIFICATION

品名規格：系列號+額定電流+額定電壓+認證符號

Model Number: Part Number+Rated Current+Rated Voltage+Approvals.

例子[example]:

3GTU，          2A，                 250V

1)系列號 Part Number，2)額定電流 Rated Current，3)額定電壓 Rated Voltage

1-2 額定電流和額定電壓                Rated Current And Voltage

額定電流：100mA~10A               Rated Current: 100mA~10A

額定電壓：250V AC                 Rated Voltage:250V AC

1-3 標示 Marking

保險絲上需有下列標示               The fuse shall have the following marking:

規格：特性符號+額定電流+額定電壓   Specification:Characteristic Symbol+Rated Voltage

100mA~800mA : _mA250V   1A~10A: _A250V

Trade mark:The Manufacture’s Logo+Part Number+Approvals

100mA~10A:

注意：對標示字樣大小沒有規定

Note:Size of the markings have not be specified.

3-2 結構 Structure

| 編號No. | 元件Component | 材質Material | 數量Quantity |
| --- | --- | --- | --- |
| 1 | 熔絲 Wire | 合金 Alloy | 1 |
| 2 | 銅帽 Cap | 黃銅鍍鎳 Brass with mickel plated | 2 |
| 3 | 本體 Body | 玻璃管 GLASS Tube | 1 |
| 4 | 引線 Lead | 鍍錫銅線 Tinned Copper wire | 2 |

2.適用範圍  SCOPE

適用於一般電子設備。Used in electronic equipment.

3.工程圖和結構  OUTLINE DRAWING AND STRUCTURE

3-1 工程圖（單位：mm）Outline Drawing and Dimensions(Unit:mm)

4.外觀及形狀

4-1 外觀：不應有破碎、明顯的汙斑

4-2 形狀：帶腳的保險絲

5.物理特性

5-1 拉力測試

沿端子的軸向施加 10N 的拉力，持續 1 分鐘。

4.APPEARANCES AND CONFIGURATION

4-1 Appearances:There shall no fragmentation and of remarkable blotch.

4-2 Configuration:Axial Leaded Fuse.

5.PHYSICAL CHARACTERISTIC

5-1 Tensile test

Keep pulling the cap of the fuse with 10N at axial direction for 1 min.

6.電氣特性

ELECTRICAL CHARACTERISTICS

熔斷/電流特性：

Acting Time/Current Characteristic

- Breaking capacity（分断能力）(Time Lag and Fast Acting)

50A                 250V                  AC

b、Electrical characteristics

Time Lag

c、Temperature character

当100%的电流下温度上升不超过75℃

When 100% of rating the temperature rise doesn’t exceed 75℃.

| Rated current | % of Ampere | Opening Rating Time |
| --- | --- | --- |
| 100mA~ 10A | 100% | 200% |
|  | 4 hours, Minimum | 60 seconds, Max. |

| Catalog Number | Rating Ampere（A） | Rating Voltage（V） | Nominal Cold Resistance (Ohms) | Nominal Melting I2t (A2 sec) | Agency Approvals |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UL | CSA | CQC |
| 3GTU-.100 | .100 | 250V | 7.5300 | 0.00250 | x | x | x |
| 3GTU-.125 | .125 |  | 2.9850 | 0.00370 | x | x | x |
| 3GTU-.200 | .200 |  | 2.5280 | 0.0161 | x | x | x |
| 3GTU-.250 | .250 |  | 1.7700 | 0.0252 | x | x | x |
| 3GTU-.300 | .300 |  | 1.1880 | 0.0332 | x | x | x |
| 3GTU-.400 | .400 |  | 0.3140 | 0.0735 | x | x | x |
| 3GTU-.500 | .500 |  | 0.2250 | 0.137 | x | x | x |
| 3GTU-.600 | .600 |  | 0.1550 | 0.482 | x | x | x |
| 3GTU-.800 | .800 |  | 0.1120 | 0.949 | x | x | x |
| 3GTU-001 | 1.00 |  | 0.0940 | 1.22 | x | x | x |
| 3GTU-01.5 | 1.50 |  | 0.0515 | 3.52 | x | x | x |
| 3GTU-01.6 | 1.60 |  | 0.0494 | 3.75 | x | x | x |
| 3GTU-002. | 2.00 |  | 0.0410 | 6.56 | x | x | x |
| 3GTU-02.5 | 2.50 |  | 0.0344 | 9.19 | x | x | x |
| 3GTU-003. | 3.00 |  | 0.0247 | 18.4 | x | x | x |
| 3GTU-004. | 4.00 |  | 0.0191 | 28.6 | x | x | x |
| 3GTU-005. | 5.00 |  | 0.0114 | 58.6 | x | x | x |
| 3GTU-006. | 6.00 |  | 0.0095 | 77.2 | x | x | x |
| 3GTU-007. | 7.00 |  | 0.0084 | 94.7 | x | x | x |
| 3GTU-008. | 8.00 |  | 0.0077 | 114.0 | x | x | x |
| 3GTU-010. | 10.0 |  | 0.0070 | 224.0 | x | x | x |

Average Time Current Curves

7.環境特性

7-1 操作溫度範圍：-5℃~60℃

7-2 保存條件

溫度：+10℃~60℃

在相對濕度≤75%下平均可存放 3 年

在非露天下，相對濕度為 95%下最多可存放 30 天。

8.焊接條件

8-1 波峰焊

焊接溫度：260±5℃

焊接時間：4 Sec. Max.

8-2 手工焊錫

焊接溫度：350±5℃

焊接時間：3 Sec. Max.

9.其他使用注意事項

如果需在保險絲上加熱縮套管，烘烤溫度與時間需參照以下規定

設定烘烤溫度：不大於 150℃ 烘烤時間：50s Max.

7.ENVIRONMENTAL SPECIFICATION

7-1 Operating Temperature: -5℃~60℃

7-2 Storage Conditions

Temperature: +10℃~60℃

On average, It can be stored below the relative humidity or 75% for 3 years.

Without dew, It can be kept under humidity of 95% for 30 days.

8.SOLDERING PAAMETERS

8-1 Wave Solder

Soldering Temperature:260±5℃

Soldering Time:4 Sec.Max.

8-2 Manual Solder

Soldering Temperature: 350±5℃

Soldering Time: 3 Sec. Max.

9.OTHER NOTICE IN USING

If heat shrink tubing was needed,the roast temperature and time should be refer to the following.

The temperature should be sel under 150℃,roast time:50s Max.